YOSYS COMMANDS
1. Getting Started

- yosys → start interactive shell

- yosys -s script.ys → run commands from a script file

- yosys -p "cmd1; cmd2; ..." → run inline commands

- help → list all commands

- help <cmd> → help for a specific command

2. Input / Output

- read_verilog file.v → load Verilog RTL

- read_blif file.blif → load BLIF netlist

- read_edif file.edif → load EDIF netlist

- read_json file.json → load JSON netlist

- write_verilog out.v → write synthesized design

- write_blif out.blif → write BLIF netlist

- write_edif out.edif → write EDIF netlist

- write_json out.json → write JSON netlist

- write_ilang out.il → write Yosys internal format

3. Design Management

- design -reset → clear all designs

- hierarchy -top <module> → set top module and resolve hierarchy

- proc → convert always-blocks to processes

- flatten → inline module instances

4. RTL Elaboration & Processing

- proc → process always blocks into logic

- opt → generic optimizations

- memory → translate memories into simpler logic

- fsm → extract and optimize finite state machines

- techmap → map generic cells to technology-specific ones

5. Optimization

- opt → basic optimizations

- opt_clean → remove unused wires and cells

- opt_const → constant propagation

- opt_reduce → logic simplification

- opt_share → share common logic

6. Technology Mapping

- abc → run ABC for logic optimization and mapping

- techmap → map internal cells to target library cells

- dfflibmap -liberty cells.lib → map flip-flops

- abc -liberty cells.lib → map logic to gates in .lib

7. Formal & Verification

- sat → SAT-based equivalence checking

- miter → create equivalence miter circuits

- equiv_make → set up equivalence checking between two designs

- equiv_simple → simplify equivalence checking

- equiv_status → check equivalence result

8. Analysis / Reports

- stat → report number of cells, wires, memory, etc.

- show → generate schematic (requires Graphviz)

- ls → list modules

- dump → dump design in ILANG format

- check → check design consistency

- tee -o file.log → log all output to file

9. Example Flow

Here’s a minimal RTL → gate flow:

yosys -p "

read_verilog mydesign.v

hierarchy -top top

proc; opt; memory; fsm; opt

techmap; opt

abc -liberty sky130.lib

write_verilog synth.v

stat

"

This will:

- Read mydesign.v

- Set top module to top

- Lower processes, memories, FSMs

- Map to target library sky130.lib

- Write out synth.v

- Print a resource usage report

10. To run

a) Open teminal on project folder

b) paste D:\oss-cad-suite\start.bat

c)  paste Yosys

d) yosys -s script.ys