VIVADO -> I need to build a tcl, batch file
Step 1. Put Your Verilog in a File

Save as:

static_key_leak.v

Step 2. Create a Vivado Project

- Open Vivado → Create Project.

- Name: keyleak_demo.

- Type: RTL Project.

- Add static_key_leak.v as a source.

- Top module = static_key_leak.

- Pick any FPGA part (e.g. xc7a100tcsg324-1, from Artix-7).

Step 3. Run Synthesis

- In Flow Navigator: Run Synthesis.

- Vivado maps your RTL to FPGA gates.

- When done, open the Utilization Report.

You’ll see something like:

- LUTs: ~8 (for the XORs).

- FFs: ~8 (for storing key_reg).

- IO buffers: 16 (8-bit input, 8-bit output).

That’s your resource footprint.

open_run synth_1

Step 4. Export Netlist

Open the TCL Console in Vivado and run:

write_verilog -force static_key_leak_netlist.v

This will produce a gate-level Verilog netlist.

Open it and you’ll see your design rewritten into LUTs, FFs, and IO buffers. Example (simplified):

LUT2 #(.INIT(4'h6)) xor1 (.I0(plaintext[0]), .I1(key_reg[0]), .O(ciphertext[0]));

FDRE key_ff0 (.C(clk), .R(reset), .D(1'b0), .Q(key_reg[0]));

VERICA